Doc Ref. No:  XXX/

Version No:

Doc Issue Date:

STANDARD OPERATING PROCEDURE

CORONAVIRUS AG RAPID TEST CASSETTE (SWAB)

TEST PROCEDURE

STANDARD OPERATING PROCEDURE

Pre – task requirements

You will need a timer/stopwatch and the tests

Standard Timings

Each test takes 16 minutes overall

| Setup: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Tests to be at room temperature and comprise of the following: |  |  |  |  |
| Workstation | Extraction Tube | Buffer Vial |  | Test Cassette |
| Sterile Swab |  |  | - Staff List and Pen - PPE for person performing the test: - Face shield - Surgical Gloves - Rubbish bin for used tests |  |
| Before use |  |  |  |  |
| Check the expiry date on the side of the carton and on the cassette packet. This date reads 2023 = year 12 = month |  |  |  |  |
| Write the lot number and expiry date on the top of your team list. Top number is lot number |  |  |  |  |

| Testing Procedure: | Notes |
| --- | --- |
| Step 1:Insert the Extraction Tube and Buffer Vials into the workstation | Ensure the tube is standing firm and reaches the bottom of the workstation |
| Step 2: Add 10 drops of solution from the Buffer vial into the Extraction Tube |  |
| Step 3: Without touching the swab with your hands, remove the swab from the packaging by peeling the top of the plastic packet downwards. | Right end of swab? |
| Step 4:Carefully insert the swab into one nostril of the team member. Roll the swab 5 times against the lining of the nostril. | The swab tip should be inserted between 2-4cm. If the swab has blood or snot on it DO NOT USE. |
| Step 5:Using the same swab repeat Step 4 with the other nostril |  |
| Step 6:Withdraw the swab and place into the extraction tube |  |
| Step 7: Roll the swab at least 6 times while pressing the head against the bottom and side of the extraction tube |  |
| Step 8:Set your timer for 1 minute leaving the swab inside the extraction tube |  |
| Step 9:After 1 minute, squeeze the tube several times from the outside to immerse the swab in the sample. |  |
| Step 10: Remove the swab from the tube and discard into the rubbish. |  |
| Step 11:Insert the dropper top/lid securely onto the top of the tube. |  |
| Step 12: Open the packet that contains the test cassette and place on a clean flat surface. | Do not open until your sample is ready to be tested |
| Step 13: Hold the tube vertically and add 4 drops to the Sample Well (labelled S) |  |
| Step 14: Start the timer for 15 minutes. |  |
| Step 15: Important: Read the results of the test at 15 minutes (Do NOT interpret the results after 20 minutes. | If test result is past 20 minutes, the test will have to be redone. |
| POSITIVE RESULT – Presence of 2 lines C and T (even if faint) |  |
| NEGATIVE RESULT – 1 single line adjacent to C |  |
| INVALID RESULT – 1 single line adjacent to T |  |
| Step 16: Record each team members test result on the record sheet. |  |

Amendment Register

| New Version No. | Issue date / Review Date | Details of Amendment / Reviewed | Approved By |
| --- | --- | --- | --- |
| 1 | 08.02.2022 | New Sop created. | Sheryl Guyton and Karyn Hitchcock |

| I have read the above sop for AG rapid test procedure and I’m happy with the contents. I believe this sop to be correct and is ready to be updated in the manuals. |  |  |
| --- | --- | --- |
| Date | Name | Signature |
|  | Cameron Johnson |  |
|  | Maree Cleal |  |
|  | Mick Brown |  |